MODEL QUESTION PAPER

Fourth Semester M. Sc. Degree  Examination (CSS)

ST4 C16: STATISTICAL QUALITY CONTROL

Time:  3 Hours                                                         	Total Weights: 30

Part A

Answer any 5 questions. Weightage 1 for each question.

- Distinguish between random causes and assignable causes in SQC.

- Justify the 3σ limits as control limits in any control chart.

- Define operating characteristic function of a control chart. What is its importance in process control?

- What is a c – chart. When and where it is used.

- Explain CUSUM charts.

- Define the terms: AQL, LTPD, Producers risk, Consumers risk.

- Distinguish between multiple sampling plans and sequential sampling plans.

- Explain the technique of curtailed inspection.

Part B

Answer any 5 questions. Weightage 2 for each question.

- Explain the terms: control limits, tolerance limits and specification limits.

- Control chart of  and R in use with the following measures.

chart : CL = 420 ,   LCL = 390,    UCL = 450.

R chart : CL = 67.05 ,   LCL = 54.15,    UCL = 61.16, d2 = 2.326

The sample size is 5. Both charts exhibit control. The quality characteristic is normally distributed. (a) What is the α – risk associated with the  chart (b) specification of the quality characteristic is 415 ± 20. What are your conclusions regarding the ability of the process to produce within specifications?

- Distinguish between defects and defectives. Explain the construction

and     operation of a p chart.

- What are acceptance sampling plans? Explain the SSP.

- Describe a procedure to derive a SSP using attributes with a

specified α and β.

- What is meant by rectifying inspection? Obtain the AOQ function

of a SSP.

- Explain the method of construction of the O. C. curves for

an attribute DSP.

- Explain briefly the salient features of Dodge- Roming tables.

Part C

Answer any 3 questions. Weightage 5 for each question.

- Explain the construction and interpretation of mean chart and

range chart.

- Describe various ways in which a control chart may be modified to

meet  special situations.

- How will you study the process capability of a production process?

What are the important indices for measuring the process capability?

- Derive the ASN and ATI functions for a DSP and draw their general

shapes.

- Describe an item by item sequential sampling plan by attributes.

Derive the acceptance and rejection lines of such a plan with a

given producers risk and consumers risk.

- The measurement X on an item follows a normal distribution with

known standard deviation. The item is considered acceptable if X

is large. Derive a SSP for a specified α and β.